SMARTWATCH MANAGER

Predictive Maintenance & Energy Optimization Platform

PHASE 1 — IIOT EDGE DEVELOPMENT

Complete Implementation, Learning & Architecture Guide

| Document Type | Phase 1 Project Plan & Technical Specification |
| --- | --- |
| Project Name | SmartWatch Manager — IIoT Predictive Maintenance Platform |
| Audience | B.Tech Student — Final Year / Portfolio / Placement Project |
| Phase Focus | ESP32 Edge Node, C/C++ Firmware, MQTT Telemetry, Data Schemas |
| Daily Study Allocation | 2 hours per day (learning + implementation) |
| Phase Duration | 7 Weeks (Weeks 1 through 7) |

Prepared As Per Industry-Grade IIoT Engineering Standards | 2025

# Table of Contents

Table of Contents	1

1. Executive Summary	1

1.1 Business Problem Being Solved	1

1.2 Phase 1 Deliverables at a Glance	1

2. Business Objective	1

2.1 Industrial Use Case — SmartWatch Manager	1

2.2 Primary Pain Points Addressed	1

2.3 Why This Matters for Your Career	1

3. Learning Roadmap	1

3.1 Concepts to Master in Phase 1	1

3.2 Weekly Learning Schedule (2 Hours / Day)	1

4. Hardware Requirements	1

4.1 Hardware Safety Notes	1

5. Software Requirements	1

5.1 Development Tools	1

5.2 Firmware Libraries (Arduino/PlatformIO)	1

6. Architecture Design — Phase 1 Edge Layer	1

6.1 Logical Architecture Diagram (Text Representation)	1

6.2 MQTT Topic Schema Design	1

6.3 JSON Telemetry Payload Schema (v1.0)	1

7. Implementation Plan	1

7.1 Step-by-Step Implementation Sequence	1

8. Deliverables	1

9. Test Cases	1

9.1 Unit Test Cases	1

9.2 Integration Test Cases	1

9.3 System / Resilience Test Cases	1

10. Success Criteria	1

11. Learning Resources	1

11.1 ESP32 & Embedded C/C++	1

11.2 MQTT & Mosquitto	1

11.3 Sensor-Specific Resources	1

12. Timeline Estimate — Gantt-Style Overview	1

12.1 Phase 1 Weekly Gantt Chart	1

12.2 Full Project Milestone Timeline (All 8 Phases)	1

13. Risks and Mitigation Strategies	1

13.1 Industry Best Practices for Phase 1	1

Security	1

Scalability	1

Reliability	1

Maintainability	1

Phase 1 Completion Checklist	1

# 1. Executive Summary

SmartWatch Manager is an end-to-end Industrial Internet of Things (IIoT) platform engineered to address two of the most economically damaging problems in modern manufacturing: unplanned machine downtime and invisible energy waste. By fusing edge computing, real-time data streaming, cloud-native backend APIs, predictive machine learning, and cross-platform dashboards into a single cohesive system, it delivers continuous, actionable intelligence about every monitored industrial asset.

Phase 1 is the foundational layer of the entire system. Without reliable, calibrated data flowing from the factory floor, every upstream component — the ML model, the dashboard, the alert engine — is meaningless. This phase establishes that trustworthy foundation by programming ESP32 microcontrollers to capture vibration, temperature, and current consumption from industrial machinery and transmit structured telemetry securely over MQTT to a local broker.

## 1.1 Business Problem Being Solved

Manufacturing industries globally lose an estimated $50 billion annually to unplanned downtime (Aberdeen Research). Phase 1 directly addresses the data acquisition gap — the root cause of why predictive systems fail to deliver: poor-quality, inconsistent, or absent sensor data.

| Without Phase 1 | With Phase 1 | Business Benefit |
| --- | --- | --- |
| No real-time sensor data | Continuous 5-second telemetry | Early fault detection |
| Manual inspections only | Automated anomaly signals | Reduced labour costs |
| No energy visibility | Per-machine current monitoring | 10-15% energy savings |
| Reactive maintenance | Data pipeline for ML input | Planned maintenance = zero surprise |

## 1.2 Phase 1 Deliverables at a Glance

- Fully functional ESP32 firmware written in C/C++ (Arduino framework)

- Sensor integration: MPU6050 (vibration), DHT22 (temperature/humidity), SCT-013 (current)

- MQTT telemetry publishing to Eclipse Mosquitto broker every 5 seconds

- Structured JSON payload schema with versioning support

- Wiring schematic and hardware bill of materials (BOM)

- Unit-tested firmware with serial monitor validation

- Complete Phase 1 technical documentation

# 2. Business Objective

## 2.1 Industrial Use Case — SmartWatch Manager

SmartWatch Manager targets small-to-medium manufacturing plants (10 to 200 machines) where dedicated industrial sensor networks (like Siemens MindSphere or GE Predix) are prohibitively expensive. It provides an affordable, open-source-compatible alternative that delivers enterprise-grade observability.

## 2.2 Primary Pain Points Addressed

| # | Pain Point | Current Industry Reality | Phase 1 Solution |
| --- | --- | --- | --- |
| 1 | Bearing Failures | Discovered only after catastrophic failure | Vibration anomaly detection via MPU6050 |
| 2 | Motor Overheating | Thermal runaway causes fires or burnout | Continuous temperature tracking via DHT22 |
| 3 | Energy Overload | Unknown per-machine power draw | Non-invasive current monitoring via SCT-013 |
| 4 | Data Silos | Sensor data not forwarded to IT systems | MQTT bridge to central broker for all consumers |

## 2.3 Why This Matters for Your Career

Phase 1 directly maps to three high-demand job roles in the 2025 tech market:

| Role | Skill Demonstrated | Average Package (India) |
| --- | --- | --- |
| IoT/Embedded Engineer | ESP32, C/C++, sensor calibration, MQTT | 8 – 20 LPA |
| IIoT Solutions Architect | Protocol design, edge computing, system architecture | 15 – 35 LPA |
| Industry 4.0 Engineer | OT/IT convergence, SCADA data modelling | 12 – 28 LPA |

# 3. Learning Roadmap

Designed for a B.Tech student allocating 2 hours per day. Each week is structured with a learning block followed by an implementation block. Never skip the learning block — implementation quality is directly proportional to conceptual clarity.

## 3.1 Concepts to Master in Phase 1

| # | Concept | Learning Hours | Impl. Hours | Difficulty |
| --- | --- | --- | --- | --- |
| 1 | ESP32 Architecture: Dual-core Xtensa LX6, GPIO, ADC, I2C, SPI | 6h | 4h | Medium |
| 2 | C/C++ for Embedded: structs, pointers, memory, non-blocking timers | 8h | 6h | Medium-High |
| 3 | MPU6050: I2C protocol, register maps, DMP, calibration offsets | 5h | 4h | Medium |
| 4 | DHT22: 1-wire protocol, humidity compensation, error handling | 3h | 2h | Easy |
| 5 | SCT-013: AC current sensing, Burden resistor, RMS calculation | 5h | 4h | Medium-High |
| 6 | MQTT Protocol: QoS levels, topic hierarchy, retained messages, LWT | 6h | 4h | Medium |
| 7 | Eclipse Mosquitto: broker config, ACL, TLS basics, Docker deployment | 4h | 3h | Medium |
| 8 | JSON Schema Design: versioning, nested payloads, validation patterns | 3h | 2h | Easy |
| 9 | Wi-Fi on ESP32: WPA2 enterprise, reconnect logic, static IP | 3h | 2h | Easy-Medium |
| 10 | Firmware Testing: Serial monitor debugging, unit test frameworks | 4h | 4h | Medium |
| TOTAL |  | 47 Hours | 35 Hours | 82 Hours |

## 3.2 Weekly Learning Schedule (2 Hours / Day)

| Week | Focus Area | Daily Activities | Hours | Output |
| --- | --- | --- | --- | --- |
| W1 | ESP32 + C++ Basics | Days 1-3: ESP32 architecture, GPIO, ADC study Days 4-5: C++ structs, pointers, FreeRTOS basics Days 6-7: PlatformIO setup + Blink LED | 14h | Dev environment ready |
| W2 | Sensor Integration | Days 1-2: MPU6050 I2C & register map Days 3-4: DHT22 1-wire wiring + code Days 5-7: Calibration + serial output working | 14h | Sensor readings on serial |
| W3 | Current Sensing | Days 1-3: SCT-013 theory + burden resistor calc Days 4-5: ADC sampling + RMS calculation Days 6-7: Calibration against known load | 14h | Current readings in Amps |
| W4 | MQTT + Mosquitto | Days 1-2: MQTT protocol deep dive (QoS, LWT) Days 3-4: Mosquitto Docker setup + MQTTX test Days 5-7: ESP32 PubSubClient integration | 14h | ESP32 publishing to broker |
| W5 | JSON Schema + Payload | Days 1-2: JSON schema design + ArduinoJson Days 3-5: Build final payload structure Days 6-7: Multi-sensor combined firmware | 14h | Complete MQTT JSON payload |
| W6 | Resilience + Security | Days 1-3: Wi-Fi reconnect logic + watchdog timer Days 4-5: Mosquitto ACL + username auth Days 6-7: LWT dead-device alert | 14h | Production-ready firmware |
| W7 | Testing + Docs | Days 1-3: Unit & integration test cases Days 4-5: Wiring schematic + BOM finalization Days 6-7: Phase 1 documentation + demo video | 14h | Phase 1 sign-off |

# 4. Hardware Requirements

All components below are selected for their availability on Indian e-commerce platforms (Robu.in, Amazon India, Mouser India, Electronicscomp). Prices are approximate as of 2025.

| # | Component | Model / Spec | Qty | Cost (INR) | Interface | Purpose & Why Selected |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Microcontroller | ESP32 DevKit v1 | 2 | ~550 | Wi-Fi/BT | Dual-core 240MHz, built-in Wi-Fi, large community, cheap. Second unit for redundancy. |
| 2 | Vibration Sensor | MPU6050 GY-521 | 2 | ~120 | I2C | 6-DoF: 3-axis accel + 3-axis gyro. Industry standard for mechanical vibration profiling. |
| 3 | Temp/Humidity | DHT22 (AM2302) | 2 | ~90 | 1-Wire | ±0.5°C accuracy, -40 to 80°C range. Adequate for motor thermal monitoring. |
| 4 | Current Sensor | SCT-013-030 (30A) | 2 | ~350 | Analog | Non-invasive AC clamp. Clamps around existing cable — no circuit modification needed. |
| 5 | Burden Resistor | 33Ω ¼W (or 68Ω for 100A variant) | 4 | ~10 | Passive | Converts SCT-013 current output to voltage for ADC reading. Critical for accuracy. |
| 6 | Status LEDs | 5mm LEDs (Red/Green/Yellow) | 6 | ~20 | GPIO | Visual feedback: Wi-Fi status, MQTT connection, alert trigger. |
| 7 | Buzzer | Active 5V Buzzer | 1 | ~30 | GPIO | Audible alarm on threshold breach. Simulates real factory alert. |
| 8 | Relay Module | 5V 2-Channel Relay | 1 | ~80 | GPIO | Simulates machine shutdown on anomaly detection — important demo feature. |
| 9 | Breadboard + Wires | 830-point + M-M/M-F jumpers | 2 sets | ~150 | — | Rapid prototyping. Use 20cm female-to-female for I2C to avoid loose connections. |
| 10 | Power Supply | 5V 2A USB Adapter or Li-Po 3.7V + TP4056 | 2 | ~200 | USB/Barrel | Stable power is non-negotiable for reliable ADC readings. USB noise = bad ADC. |
| 11 | Pull-up Resistors | 4.7kΩ ¼W for I2C + 10kΩ for DHT22 | 10 | ~15 | Passive | Required for I2C SDA/SCL lines and DHT22 data pin. |
| 12 | Decoupling Caps | 100nF + 100µF electrolytic | 5 each | ~30 | Passive | Filters power rail noise — especially important near motor test environments. |
| TOTAL (approx.) |  |  |  | ₹1,645 |  |  |

## 4.1 Hardware Safety Notes

- NEVER connect the SCT-013 to mains voltage (230V AC) without proper insulation. Always use a low-voltage test transformer during development.

- Add a 100µF capacitor across the ESP32 3.3V rail to prevent brownouts during Wi-Fi transmission bursts.

- I2C pull-up resistors (4.7kΩ) are mandatory — the MPU6050 module often omits them.

- Use shielded cable for SCT-013 signal wire to prevent EMI pickup from motor fields.

# 5. Software Requirements

## 5.1 Development Tools

| Tool | Version | Category | Purpose |
| --- | --- | --- | --- |
| VS Code | Latest | IDE | Primary development environment with IntelliSense, Serial Monitor, Git |
| PlatformIO IDE Extension | 6.x | Framework | Manages ESP32 toolchain, libraries, build system, upload pipeline |
| Arduino ESP32 Core | 3.0.x | SDK | Arduino-compatible API layer over Espressif IDF for ESP32 |
| Eclipse Mosquitto | 2.0.x | MQTT Broker | Local MQTT broker; run via Docker for easy management |
| Docker Desktop | Latest | Container Runtime | Runs Mosquitto as container; prepares for multi-service Phase 2+ |
| MQTTX | Latest | MQTT Client | GUI-based MQTT client to inspect, subscribe, and publish test messages |
| Git + GitHub | Latest | Version Control | All firmware code must be version controlled from Day 1 |

## 5.2 Firmware Libraries (Arduino/PlatformIO)

| Library | Maintainer | Install via | Use Case |
| --- | --- | --- | --- |
| PubSubClient | Nick O'Leary | PlatformIO libs | MQTT client for ESP32; supports QoS 0 and 1 |
| ArduinoJson | Benoit Blanchon | PlatformIO libs | Build, serialize, and parse JSON payloads efficiently |
| Adafruit MPU6050 | Adafruit | PlatformIO libs | High-level I2C driver for MPU6050 with calibration support |
| DHT Sensor Library | Adafruit | PlatformIO libs | Read temperature and humidity from DHT22 with error codes |
| EmonLib | OpenEnergyMonitor | PlatformIO libs | RMS calculation for AC current from SCT-013 ADC samples |
| WiFi.h (built-in) | Espressif | Arduino ESP32 Core | Wi-Fi connection management, static IP, reconnect handler |

# 6. Architecture Design — Phase 1 Edge Layer

The Phase 1 architecture follows the Observe-Compute-Transmit pattern which is the foundational pattern for all IIoT edge nodes. Understanding this pattern deeply will make Phases 2 through 8 dramatically easier to reason about.

## 6.1 Logical Architecture Diagram (Text Representation)

┌─────────────────────────────────────────────────────────────────────┐   │                     FACTORY FLOOR (Physical)                        │   │  ┌──────────────┐   ┌──────────────┐   ┌──────────────────────┐   │   │  │  MPU6050     │   │    DHT22      │   │     SCT-013          │   │   │  │  (Vibration) │   │  (Temp/Humid) │   │  (AC Current Clamp) │   │   │  │  I2C: 0x68   │   │  1-Wire GPIO  │   │  Analog ADC GPIO34  │   │   │  └──────┬───────┘   └──────┬────────┘   └──────────┬───────────┘   │   │         │                  │                        │               │   │         └──────────────────┼────────────────────────┘               │   │                            ▼                                         │   │              ┌─────────────────────────────┐                        │   │              │         ESP32 DevKit v1       │                        │   │              │  ┌─────────────────────────┐ │                        │   │              │  │  1. Sampling Scheduler  │ │                        │   │              │  │     (Non-blocking timer) │ │                        │   │              │  ├─────────────────────────┤ │                        │   │              │  │  2. Signal Conditioning  │ │                        │   │              │  │     (RMS, offset, units) │ │                        │   │              │  ├─────────────────────────┤ │                        │   │              │  │  3. Threshold Checker   │ │                        │   │              │  │     (Edge alerting)     │ │                        │   │              │  ├─────────────────────────┤ │                        │   │              │  │  4. JSON Payload Builder│ │                        │   │              │  │     (ArduinoJson)       │ │                        │   │              │  ├─────────────────────────┤ │                        │   │              │  │  5. MQTT Publisher      │ │                        │   │              │  │     (PubSubClient)      │ │                        │   │              │  └─────────────────────────┘ │                        │   │              └──────────────┬────────────────┘                        │   │                             │  Wi-Fi (WPA2) TCP/IP                    │   └─────────────────────────────┼─────────────────────────────────────────┘                                 ▼                 ┌──────────────────────────────────┐                 │   Eclipse Mosquitto MQTT Broker   │                 │   (Docker Container: Port 1883)   │                 │                                   │                 │  Topics:                          │                 │  smartwatch/factory_01/machine_01 │                 │  smartwatch/factory_01/machine_02 │                 │  smartwatch/+/+/alerts            │                 └──────────────┬───────────────────┘                                │            ┌───────────────────┼──────────────────────┐            ▼                   ▼                      ▼     [MQTTX Client]     [Phase 2: Kafka]     [Phase 3: Backend API]     (Testing only)     (Coming Next)         (Coming Next)

## 6.2 MQTT Topic Schema Design

Topic hierarchy follows the industry-standard pattern: {project}/{location}/{asset}/{data_type}. This design enables fine-grained subscriptions and scales to thousands of devices.

| MQTT Topic | QoS Level | Description |
| --- | --- | --- |
| smartwatch/{factory_id}/{machine_id}/telemetry | QoS 1 | Main sensor payload — vibration, temperature, current. Published every 5 seconds. |
| smartwatch/{factory_id}/{machine_id}/alerts | QoS 2 | Threshold breach alerts — exactly-once delivery. High criticality. |
| smartwatch/{factory_id}/{machine_id}/status | QoS 1 | Device heartbeat — online/offline indicator. Published every 30 seconds. |
| smartwatch/{factory_id}/{machine_id}/lwt | QoS 1 | Last Will and Testament — auto-published by broker on unexpected disconnect. |
| smartwatch/cmd/{machine_id}/config | QoS 1 | Command topic — for remote configuration of sampling rate, thresholds (Phases 3+). |

## 6.3 JSON Telemetry Payload Schema (v1.0)

{   "schema_version": "1.0",   "device_id": "ESP32-FACTORY01-M01",   "factory_id": "FACTORY_01",   "machine_id": "MACHINE_01",   "timestamp_utc": "2025-07-15T10:30:05Z",   "uptime_seconds": 3600,   "sensors": {     "vibration": {       "accel_x_g": 0.023,       "accel_y_g": 0.018,       "accel_z_g": 1.001,       "gyro_x_dps": 0.12,       "gyro_y_dps": -0.08,       "gyro_z_dps": 0.05,       "rms_accel_g": 0.029,       "status": "OK"     },     "thermal": {       "temperature_c": 42.5,       "humidity_pct": 65.2,       "status": "OK"     },     "power": {       "current_amps": 7.83,       "voltage_v_nominal": 230,       "apparent_power_va": 1800.9,       "status": "OK"     }   },   "alerts": {     "active_alerts": [],     "alert_count": 0   },   "network": {     "rssi_dbm": -62,     "ip_address": "192.168.1.105"   } }

# 7. Implementation Plan

The implementation follows a strict Build-Validate-Integrate cycle. Do not move to the next component until the current one passes its validation test.

## 7.1 Step-by-Step Implementation Sequence

| # | Wk | Task | Implementation Detail | Validation Gate |
| --- | --- | --- | --- | --- |
| 1 | 1 | Environment Setup | Install VS Code, PlatformIO, ESP32 board package. Create GitHub repo with proper .gitignore. | Upload Blink sketch. LED flashes at 1Hz. |
| 2 | 1 | MPU6050 Integration | Wire I2C (SDA=GPIO21, SCL=GPIO22). Install Adafruit MPU6050 lib. Read raw accel/gyro, print to Serial. | Serial shows X/Y/Z accel values. No 0.00 flatlines. |
| 3 | 2 | MPU6050 Calibration | Place MPU6050 flat. Run calibration sketch to compute offsets. Store offsets in EEPROM or #define constants. | accel_z = ~1.0g at rest. Offset error < 0.01g. |
| 4 | 2 | DHT22 Integration | Wire DATA pin to GPIO4 with 10kΩ pull-up to 3.3V. Install DHT library. Read temp + humidity with error handling. | Serial shows temp and humidity. NaN errors handled gracefully. |
| 5 | 3 | SCT-013 Integration | Connect SCT-013 to GPIO34 ADC via 33Ω burden + voltage divider. Install EmonLib. Sample 1480 cycles for RMS. | Reading within ±5% of reference clamp meter. |
| 6 | 3 | Mosquitto Docker Setup | Run: docker run -d --name mosquitto -p 1883:1883 -p 9001:9001 eclipse-mosquitto. Create mosquitto.conf with allow_anonymous true for dev. | MQTTX connects to localhost:1883. Can publish/subscribe. |
| 7 | 4 | ESP32 Wi-Fi + MQTT | Use PubSubClient with WiFiClient. Implement connect() function with retry loop. Test with MQTTX subscribing to test topic. | Message appears in MQTTX within 1 second of publish. |
| 8 | 5 | JSON Payload Builder | Use ArduinoJson StaticJsonDocument<512>. Build nested JSON with all sensor values. Serialize to char buffer. Publish to telemetry topic. | MQTTX shows valid JSON. All fields present. Parse without errors. |
| 9 | 5 | Multi-Sensor Fusion | Implement non-blocking millis() timer. All 3 sensors sampled in same loop iteration. Single unified payload published. | Payload published every 5s with all 3 sensor groups populated. |
| 10 | 6 | Resilience Logic | Add Wi-Fi reconnect on disconnect. Add MQTT reconnect loop. Configure LWT message. Add hardware watchdog timer (esp_task_wdt_init). | Pull Wi-Fi plug, re-plug. Device reconnects within 30 seconds. |
| 11 | 6 | Mosquitto Security | Create passwordfile with mosquitto_passwd. Set allow_anonymous false. Update ESP32 credentials. Test ACL restrictions. | Anonymous connections rejected by broker. |
| 12 | 7 | Complete Testing + Docs | Execute all test cases from Section 9. Create wiring schematic in Fritzing. Write README with setup instructions. | All 15 test cases pass. README allows fresh setup in < 30 minutes. |

# 8. Deliverables

| # | Deliverable | Format / Location | Description |
| --- | --- | --- | --- |
| D1 | Firmware Source Code | GitHub: /src/main.cpp | Complete ESP32 firmware: sensor reading, JSON serialization, MQTT publish, resilience logic. |
| D2 | Sensor Driver Modules | GitHub: /src/sensors/ | Individual .cpp/.h files for MPU6050, DHT22, SCT-013 drivers — reusable, testable. |
| D3 | MQTT Publisher Module | GitHub: /src/mqtt/ | Encapsulates connection management, reconnect, publish, LWT configuration. |
| D4 | JSON Schema Definition | GitHub: /docs/payload_schema_v1.json | Formal JSON Schema spec for payload validation by downstream consumers (Phase 2+). |
| D5 | Wiring Schematic | Fritzing .fzz + exported PNG | Visual circuit diagram showing all sensor connections, resistors, and power rails. |
| D6 | Hardware BOM | GitHub: /docs/BOM.csv | Bill of Materials with quantities, specifications, and supplier links. |
| D7 | Docker Compose File | GitHub: /infra/docker-compose.yml | Mosquitto broker configuration with persistent volume, authentication, and port bindings. |
| D8 | Mosquitto Config | GitHub: /infra/mosquitto/mosquitto.conf | Broker configuration: ACL, authentication, persistence settings. |
| D9 | Test Report | GitHub: /docs/Phase1_Test_Report.md | Completed test execution records for all 15 test cases with pass/fail status. |
| D10 | README + Setup Guide | GitHub: /README.md | Step-by-step setup guide: wiring, flashing, broker setup, verification steps. |
| D11 | Demo Video | YouTube (unlisted) + README link | 2-3 minute screen + physical demo showing sensor readings flowing into MQTTX in real time. |

# 9. Test Cases

All test cases must be executed and documented before Phase 1 is considered complete. Record pass/fail and any observed deviations.

## 9.1 Unit Test Cases

| Test ID | Objective | Input | Expected Result | Pass Criteria | Status |
| --- | --- | --- | --- | --- | --- |
| TC-U01 | MPU6050 boot and detection | Power on ESP32 with MPU6050 wired on I2C | Serial prints I2C address 0x68 found. No error code. | Address detected | PENDING |
| TC-U02 | Accel calibration accuracy | MPU6050 placed flat on level surface | accel_z reads 1.00g (±0.02g). accel_x, accel_y < 0.03g. | Z = 1.00g ± 0.02g | PENDING |
| TC-U03 | Vibration detection | Tap the breadboard lightly while reading | Accel RMS value spikes above 0.5g within one reading cycle. | RMS > 0.5g on tap | PENDING |
| TC-U04 | DHT22 valid reading | DHT22 at room temperature (~25°C) | Temperature between 20-35°C. Humidity between 30-80%. No NaN. | No NaN. In range. | PENDING |
| TC-U05 | DHT22 error handling | Disconnect DHT22 data pin during runtime | Serial prints DHT ERROR. status field set to ERROR. No firmware crash. | No crash. Error flagged. | PENDING |
| TC-U06 | SCT-013 zero current | SCT-013 clamp not around any cable | current_amps reads < 0.3A (noise floor) | < 0.3A idle | PENDING |
| TC-U07 | SCT-013 known load accuracy | Clamp SCT-013 around cable of known 100W bulb at 230V = ~0.43A | current_amps reads between 0.38A and 0.48A (±10%) | Within ±10% of reference | PENDING |

## 9.2 Integration Test Cases

| Test ID | Objective | Input | Expected Result | Pass Criteria | Status |
| --- | --- | --- | --- | --- | --- |
| TC-I01 | MQTT broker reachability | ESP32 powers on, Wi-Fi available, Mosquitto running on port 1883 | Serial prints MQTT Connected. Broker logs show new client connection. | Connected within 10s | PENDING |
| TC-I02 | JSON payload delivery | All sensors wired and working. MQTTX subscribed to telemetry topic. | MQTTX receives JSON message every 5 seconds (±0.5s). JSON validates against schema. | Valid JSON every 5s | PENDING |
| TC-I03 | All sensors in single payload | Normal operation with all sensors connected | Payload contains vibration, thermal, and power sections. No missing fields. | All 3 sections present | PENDING |
| TC-I04 | MQTT publish latency | Timestamp message at ESP32, measure time to MQTTX receipt | End-to-end latency < 500ms on local Wi-Fi network. | < 500ms | PENDING |

## 9.3 System / Resilience Test Cases

| Test ID | Objective | Input | Expected Result | Pass Criteria | Status |
| --- | --- | --- | --- | --- | --- |
| TC-S01 | Wi-Fi reconnect | Disconnect Wi-Fi router for 60 seconds then reconnect | Device detects disconnect, enters reconnect loop, resumes publishing after reconnect. No reboot required. | Reconnect < 30s | PENDING |
| TC-S02 | MQTT broker reconnect | Stop Docker Mosquitto container for 30s then restart | LWT message appears in broker logs. Device reconnects and resumes publishing after broker restart. | LWT fired + Reconnect OK | PENDING |
| TC-S03 | 24-hour stability test | Run ESP32 continuously for 24 hours | No memory leak (heap stays stable). No reboot events. Message count = approx. 17,280. | No crash in 24h | PENDING |
| TC-S04 | Threshold alert trigger | Artificially set temperature threshold to 25°C (room temp will exceed it) | Alert published to .../alerts topic with correct machine ID and threshold value. | Alert published correctly | PENDING |

# 10. Success Criteria

Phase 1 is considered COMPLETE only when ALL of the following criteria are met and documented:

| # | Acceptance Criterion | Measurement Method | Threshold | Status |
| --- | --- | --- | --- | --- |
| SC1 | Sensor data published to MQTT broker | MQTTX subscription + timestamp comparison | Every 5s ±0.5s | NOT MET |
| SC2 | JSON payload schema valid and complete | JSON Schema validator (jsonschema Python lib) | 0 validation errors | NOT MET |
| SC3 | Current sensor reading accuracy | Compare with calibrated clamp meter on known load | Error < ±10% | NOT MET |
| SC4 | Wi-Fi reconnection without manual reboot | Disconnect and reconnect router, observe Serial Monitor | < 30 seconds | NOT MET |
| SC5 | Mosquitto running with username/password auth | Attempt anonymous connection — must be rejected | 100% rejection | NOT MET |
| SC6 | 24-hour continuous operation with no crash | Serial monitor + message count validation | 0 unintended reboots | NOT MET |
| SC7 | All 15 test cases executed and passed | Test Report document in GitHub /docs | 15/15 PASS | NOT MET |
| SC8 | GitHub repo with clean commits, README, and BOM | Peer review of repository structure | Industry-standard folder structure | NOT MET |

# 11. Learning Resources

All resources below are either free or have free tiers. Prioritize in the order listed. Do not buy paid courses until you have exhausted the free tier.

## 11.1 ESP32 & Embedded C/C++

| Type | Resource | Link / Platform | Why Use It |
| --- | --- | --- | --- |
| YouTube | ESP32 Series by DroneBot Workshop | youtube.com/@dronebotworkshop | Best visual explanations of GPIO, I2C, ADC, Wi-Fi on ESP32 |
| Docs | Espressif ESP-IDF Programming Guide | docs.espressif.com | Official reference for all ESP32 hardware registers and APIs |
| YouTube | Random Nerd Tutorials — ESP32 | randomnerdtutorials.com | Project-based tutorials: MQTT, sensors, deep sleep. Excellent for beginners. |
| Book | Programming ESP32 by Neil Kolban (Free PDF) | esp32.com/viewtopic.php | Comprehensive 500+ page reference. Chapter on FreeRTOS is outstanding. |
| GitHub | espressif/arduino-esp32 (Official Arduino Core) | github.com/espressif/arduino-esp32 | Study the examples/ folder. Real production code patterns. |

## 11.2 MQTT & Mosquitto

| Type | Resource | Link / Platform | Why Use It |
| --- | --- | --- | --- |
| Docs | MQTT Essentials Series by HiveMQ | hivemq.com/mqtt-essentials/ | Best written MQTT guide available. Covers all QoS levels, LWT, retained messages. |
| YouTube | Andreas Spiess — MQTT Deep Dive | youtube.com/@AndreasSpiess | Practical IoT expert. Excellent Mosquitto + ESP32 integration videos. |
| Docs | Eclipse Mosquitto Official Documentation | mosquitto.org/documentation/ | mosquitto.conf reference. Essential for ACL and authentication setup. |
| GitHub | knolleary/pubsubclient — ESP32 MQTT library | github.com/knolleary/pubsubclient | Read the examples. Understand QoS callback patterns. |

## 11.3 Sensor-Specific Resources

| Sensor | Resource | Link / Platform | Key Topic to Study |
| --- | --- | --- | --- |
| MPU6050 | MPU-6050 Product Specification Rev 3.4 | invensense.tdk.com | Register map, DMP, I2C multi-master mode, FIFO buffer |
| MPU6050 | Jeff Rowberg MPU6050 Library + DMP Tutorial | github.com/jrowberg/i2cdevlib | The gold-standard Arduino MPU6050 implementation |
| SCT-013 | OpenEnergyMonitor — CT Sensor Guide | learn.openenergymonitor.org | Burden resistor calculation, biasing circuit, RMS math |
| EmonLib | EmonLib GitHub + calcIrms() explanation | github.com/openenergymonitor/EmonLib | How 1480-sample RMS calculation works. Critical for accuracy. |

# 12. Timeline Estimate — Gantt-Style Overview

## 12.1 Phase 1 Weekly Gantt Chart

| Task | W1 | W2 | W3 | W4 | W5 | W6 | W7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Environment Setup + Arduino Basics | ■ |  |  |  |  |  |  |
| ESP32 Firmware Fundamentals | ■ |  |  |  |  |  |  |
| MPU6050 Integration + Calibration |  | ■ |  |  |  |  |  |
| DHT22 Integration |  | ■ |  |  |  |  |  |
| SCT-013 Current Sensor |  |  | ■ |  |  |  |  |
| Mosquitto Docker Setup |  |  | ■ |  |  |  |  |
| Wi-Fi + MQTT (PubSubClient) |  |  |  | ■ |  |  |  |
| LWT + Reconnect Logic |  |  |  | ■ |  |  |  |
| JSON Schema + ArduinoJson |  |  |  |  | ■ |  |  |
| Multi-Sensor Unified Payload |  |  |  |  | ■ |  |  |
| Resilience + Security Hardening |  |  |  |  |  | ■ |  |
| Mosquitto ACL + Auth |  |  |  |  |  | ■ |  |
| Complete Test Execution |  |  |  |  |  |  | ■ |
| Documentation + Demo Video |  |  |  |  |  |  | ■ |
| Phase 1 Review & Sign-off |  |  |  |  |  |  | ■ |

## 12.2 Full Project Milestone Timeline (All 8 Phases)

| Ph | Phase Name | Duration | Calendar Weeks | Key Milestone |
| --- | --- | --- | --- | --- |
| P1 | IIoT Edge Development | 7 Weeks | Weeks 1–7 | ESP32 publishing multi-sensor JSON to MQTT broker |
| P2 | Data Streaming & Ingestion | 5 Weeks | Weeks 8–12 | Kafka consuming MQTT, Node-RED pipeline, data reaches PostgreSQL |
| P3 | Backend API Development | 5 Weeks | Weeks 13–17 | FastAPI/Node.js REST + WebSocket API with JWT auth running in Docker |
| P4 | Relational Database (PostgreSQL) | 4 Weeks | Weeks 18–21 | RBAC schema, TimescaleDB hypertable, optimized time-series queries |
| P5 | Machine Learning Pipeline | 6 Weeks | Weeks 22–27 | LSTM RUL model + XGBoost fault classifier deployed as FastAPI microservice |
| P6 | Web Frontend (React) | 5 Weeks | Weeks 28–32 | React dashboard with live WebSocket, factory canvas, RBAC-gated views |
| P7 | Mobile Application (Flutter) | 5 Weeks | Weeks 33–37 | Flutter app with biometric login, FCM push alerts, acknowledge workflow |
| P8 | DevOps, Cloud & Thesis | 4 Weeks | Weeks 38–41 | Full system on AWS/Azure via Docker Compose + GitHub Actions CI/CD + final thesis |
| TOTAL | Complete SmartWatch Manager Platform | 41 Weeks | ~10 Months | Industry-grade IIoT platform — portfolio-ready |

# 13. Risks and Mitigation Strategies

| # | Risk | Prob. | Impact | Mitigation Strategy |
| --- | --- | --- | --- | --- |
| R1 | MPU6050 I2C address conflict (0x68 vs 0x69) | Medium | Low | Set AD0 pin HIGH on second MPU6050 to change address to 0x69. Only one per I2C bus at 0x68. |
| R2 | SCT-013 ADC noise / inaccurate readings | High | Medium | Use ESP32 GPIO34 (no internal pull-up noise). Add 100µF cap on power rail. Calibrate EmonLib calibrationFactor against known load. |
| R3 | Wi-Fi instability causing MQTT disconnect loops | Medium | Medium | Implement exponential backoff in reconnect logic. Add hardware watchdog (esp_task_wdt). Test with router at 5m+ distance. |
| R4 | DHT22 returning NaN intermittently | High | Low | Add retry logic (3 attempts with 100ms delay). Minimum 2-second interval between DHT22 reads. Use last-known-good value as fallback. |
| R5 | ArduinoJson buffer overflow / memory corruption | Medium | High | Use ArduinoJson capacity calculator tool. Use StaticJsonDocument with size verified at compile time. Monitor heap with ESP.getFreeHeap(). |
| R6 | MQTT message loss during Wi-Fi transitions | Low | Medium | Use QoS 1 for telemetry (at-least-once). Buffer last reading in global variable for immediate re-publish after reconnect. |
| R7 | Timeline slippage due to learning curve | High | Medium | Built 1-week buffer between Phase 1 and Phase 2. Prioritize working code over perfect code — document what you plan to refactor. |
| R8 | Hardware component damaged during prototyping | Low | Medium | Order 2x quantity for all critical components (ESP32, MPU6050). Use current-limiting resistors on all GPIO outputs. Never exceed 3.3V on ESP32 inputs. |

## 13.1 Industry Best Practices for Phase 1

### Security

- Never hardcode Wi-Fi passwords or MQTT credentials in source code — use a config.h file excluded from Git via .gitignore.

- Enable Mosquitto password-based authentication before moving to Phase 2. Anonymous access is only acceptable in isolated local development.

- Plan for TLS/SSL encryption on MQTT connections in Phase 2 (port 8883) even if not implemented in Phase 1.

### Scalability

- Design the MQTT topic schema with multi-factory and multi-machine hierarchy from Day 1. Changing topic schemas later breaks all downstream consumers.

- The JSON schema version field (schema_version: '1.0') is critical — it enables non-breaking upgrades by downstream systems.

### Reliability

- The watchdog timer is non-negotiable in a production IoT device. ESP32 hardware WDT + software WDT should both be configured.

- Implement a 'heartbeat' topic separate from telemetry — allows the backend to distinguish 'no data' from 'device offline'.

### Maintainability

- Separate concerns into individual .cpp/.h module files from Day 1: sensor_mpu6050.cpp, sensor_dht22.cpp, mqtt_client.cpp, payload_builder.cpp. Monolithic main.cpp becomes unmaintainable past 200 lines.

- Use semantic versioning for firmware: FIRMWARE_VERSION '1.0.0'. Include this in every MQTT payload for remote diagnostics.

# Phase 1 Completion Checklist

Use this checklist as your daily progress tracker. Check off each item only when it is working, committed to Git, and documented.

| Done | Checklist Item | Week Target | Phase Gate |
| --- | --- | --- | --- |
| ☐ | VS Code + PlatformIO installed. Blink sketch running on ESP32. | W1 | Setup |
| ☐ | MPU6050 I2C readings on Serial Monitor. Calibration offsets applied. | W2 | Sensor |
| ☐ | DHT22 temperature and humidity reading. Error handling tested. | W2 | Sensor |
| ☐ | SCT-013 current reading calibrated. Within 10% of reference. | W3 | Sensor |
| ☐ | Mosquitto running in Docker. MQTTX can connect and subscribe. | W3 | Broker |
| ☐ | ESP32 Wi-Fi connects. MQTT client connects to broker. | W4 | Network |
| ☐ | LWT configured. Device offline status visible in MQTTX. | W4 | Network |
| ☐ | JSON payload built with ArduinoJson. All sensor fields present. | W5 | Payload |
| ☐ | Payload published every 5 seconds. MQTTX receiving correctly. | W5 | Integration |
| ☐ | Wi-Fi reconnect logic tested. Reconnects within 30 seconds. | W6 | Resilience |
| ☐ | MQTT broker reconnect tested. Resumes publishing after broker restart. | W6 | Resilience |
| ☐ | Mosquitto password auth enabled. Anonymous connections rejected. | W6 | Security |
| ☐ | All 15 test cases executed. Test report written in /docs. | W7 | Testing |
| ☐ | Wiring schematic saved in Fritzing and exported as PNG. | W7 | Docs |
| ☐ | GitHub repo organized: /src, /docs, /infra with clean README. | W7 | Docs |
| ☐ | Demo video recorded (2-3 min). Link added to README. | W7 | Handoff |
| ☐ | Phase 1 accepted. Phase 2 planning commenced. | W7 | ✦ PHASE GATE |

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

SmartWatch Manager — Phase 1: IIoT Edge Development

Designed to Industry 4.0 Standards  |  Final Year Project  |  Portfolio Edition  |  2025